Intermediate Reports on DSP Optimizations

Group Name                      : crypto group 1

Team member 1               : Axel Vanraes

Team member 2               : Michaël Raes

Michaël Raes                                                                           Axel Vanraes

Include the following details in your reports.

         Optimizations tried and carried out. Discuss its impact on the cycles at the function level and total code level. Also present your reasoning about the impact of the optimization which you observed.

 	Even if an optimization does not improve the result, include and discuss it.

The following table is an overview of all performed optimizations. (KA=Key Agreement, ED=Encrypt/Decrypt)
The input data (a 30 byte buffer) is defined in a variable (during phase 1)

|  | Major Optimization | Functions Effected | Total Cycles* |  | Change (in %) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Before(KA/ED) | After |  |
| Base Code | N/A | N/A | 537M / 49K | N/A | N/A |
| Session 1 | S1.keygetters | get_slave_privkey get_slave_pubkey get_slave_pubkey get_master_privkey get_master_pubkey get_master_modulus | 537M / 49K | 537M / 49K | 0.00025 |
|  | S1.modinv | mod_inverse | 503K | 400 | 99.92% |
| Session 2 | S2.modinv | Mod_inverse | 503K | 1190 |  |
|  | S2.Must_It | Monmult | 535M | 535M | 0 |
|  | S2.unroll | Monmult | 535M | 513M | 4.11% |
| Session 3 | : | : |  |  |  |
| : | : | : |  |  |  |

# Report 1

Lab date                              : 22/04, 2016

Submission date               : 23/04, 2016

Submission time              : 4 PM

## Logs:

This session was used for profiling the base code and identifying the most obvious optimizations. It is clear from the profiling results that the only significant gain is to be expected from optimizations to the Montgomery multiplication code. Minor improvements can be expected from internally restructuring code to avoid the needless endianness conversions we do.

Some final modifications were made on the functionality (EOT, reset of symmetric key, …) this may add some cycles to the asymmetric part but the effect is negligible.

## Profile results:
(Subdirectory names are abbreviated. as=asymmetric, ci=ciphers, pm=packman, …)
(monmult = Montgomery Multiplication, monexp = Montgomery Exponentiation)

| Function Name | File Name | Total no. of calls | Exclusive Count Total* | After (This Session) | %Change | Inclusive Count Total* | After (This Session) | % Change | Optimization carried out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| monmult | as/monmult.c | 4057 | 534M |  |  | 534.6M |  |  |  |
| monexp | as/monexp.c | 9 | 104K |  |  | 534.7M |  |  |  |
| mod_inverse | as/monmult.c | 4057 | 503K | 400 | 99.92% | 503K | 400 | 99.92% | S1.modinv |
| Multiple | sts/*.c |  |  |  |  |  |  |  | S1.keygetters |

Optimization details: (Optimization names follow the format “S(session number).keyword”.)
S1.modinv is the removing of the unnecessary calculations of the least significant byte of the inverse of the modulus. This was performed for every Montgomery Multiplication, whilst it only has to be calculated once for every used modulus. There are 3 moduli in this application (master, slave and Diffie-Hellman) so this gives an improvement of (4057-3)/4057 = 99.9% or about 500K cycles, which is still incomparable to the heavyweight that is Montgomery Multiplication.

In S1.keygetters we define the keys for master and slave directly in the right format (Little Endian), instead of using the given (Big Endian) vectors from Toledo. This allows us to remove the “getter”-functions that took some unnecessary cycles. We expected an insignificant improvement from this, but the situation has actually worsened: it now takes more cycles. We don’t really understand what happened. If we add getters that makes a saving the “getters” again the total cycles increases again with 168 cycles/getter. Master and slave use four of 8*168=1344 cycles or 0.00025%.

## Discussion (important)

         We expect the change from 8 to 32 bit in the Montgomery Multiplication algorithm to give a huge performance increase. We’ll try to implement this by next session. Subsequently it can be interesting to rewrite parts of ‘monmult’ in assembly as the function takes roughly 132k cycles per call.

- The performance of the real-time component (the encryption/decryption using AES-OCB) is sufficient, so we will focus on the asymmetric crypto calculations.

# Report 2

Lab date                              : 28/04, 2016

Submission date               : 29/04, 2016

Submission time              : 4 PM

## Logs:

We had to do some extra changes to preserve the same functionality as before optimization S1.modinv. The result is that the total cycle count of modular inversion is now about 1k instead of the 400 we previously stated. However, the amount of calls is still reduced from 4057 to 14.

The work on changing Montgomery Multiplication from 8-bit word size to 16-bit continues, but it takes more time than previously estimated. We try to finish it by next session.

We have also started looking at the list of “mandatory/recommended” optimizations to see if they apply in our case, specifically the use of the different pragma’s. We tried using the MUST_ITERATE pragma but the result was quasi nonexistent. UNROLL gave better results.

## Profile results:
(Subdirectory names are abbreviated. as=asymmetric, ci=ciphers, pm=packman, …)
(monmult = Montgomery Multiplication, monexp = Montgomery Exponentiation)

| Function Name | File Name | Total no. of calls | Exclusive Count Total* | After (This Session) | %Change | Inclusive Count Total* | After (This Session | % Change | Optimization carried out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mod_inverse | as/monmult.c | 14 | 503K | 1190 | - | 503K | 1190 |  | S2.modinv |
| monmult | as/monmult.c | 4057 |  |  | 0 |  |  | 0 | S2.Must_It |
| monmult | as/monmult.c | 4057 | 534M | 510M | 5% | 534M | 510M | 5% | S2.unroll |

Optimization details: (Optimization names follow the format “S(session number).keyword”.)
S2.modinv is an update to the changes we did in S1.modinv to iron out some bugs.
In S2.Must_It we tried to use the MUST_ITERATE pragma for some of our Montgomery Multiplication loops, this gave no significant result. We’re still trying to figure out how to use pragma’s effectively.
S2.unroll, where we did some loop unrolling in the Montgomery Multiplication, this gave a reduction of about 24M cycles, or about 5%.

## Discussion (important)

- We will change the Montgomery word size from 8 to 16, instead of the 32 previously stated, because multiplications with 16 bit words are the most efficient on the target DSP.

- An analysis of our Montgomery Multiplication (FIPS) algorithm requires 2s 2 + s multiplications, 6s 2 + 2s + 2 additions, 9s 2 + 8s + 2 reads, and 5s 2 + 8s + 1 writes, where s is the amount of “Montgomery words” in the modulus. For the non-optimized code s is 156, however if we change the word size to 16 bit, s is only 78. The vast possibility for cycle reduction is apparent.